# Z-Wave RF Range Testing Executive Summary

March 2026

A series of field tests were performed measuring the Line-Of-Sight (LoS) RF Range of Z-Wave in various modes. The threshold for the range is 100% error-free communication, fully encrypted. The absolute maximum range where we can occasionally get a message thru would be much further. However, Z-Wave demands reliable communication, so the threshold is set at the point before significant degradation of the channel. The focus of these tests is on the Z-Wave Alliance Reference Application Design (ZRAD) kit. The results are:

- ZRAD to ZRAD in ZWLR maximum LoS range =

- ZRAD to ZRAD in US Z-Wave Mesh maximum LoS range =

Details are below.

Plan for RF testing March 2026:

- Location – East coast of NH/MA across the ocean

  - ED will travel up Plum Island – road is not LoS as it is behind dunes

    - on beach which is LoS

  - Then Rt 1A along Salisbury beach which is again not LoS

    - On beach is LoS

  - Then Rt 1A in Hampton NH again no LoS

    - On beach is LoS

    - Ocean Blvd is LoS

    - North end of North Beach is also LoS at times

    - End is Ordiorne point park about 25miles from Rockport

  - Controller locations:

    - Hellcat Observation Tower – plum island – behind dunes so might not be LoS

      - It is elevated about 30’

    - Cranes Beach platform on Dune – short walk from the parking lot to the top of a dune with a small platform that has LoS up the beach

      - Not ideal as the end of Plum Island obstructs LoS

      - But it is elevated

    - UMASS Gloucester Marine Station – LoS to the entire 25mi path above

      - Should be able to park on the edge of the parking lot with direct view of bay

      - Might want to contact UMass to get an OK?

    - Duley St, Harvey Reservation – A little further out on the point which should have LoS to the entire path

    - Ocean View point – at the end of the point – long walk from the parking lot (1/2mi)

  - These mostly require driving with the ED out the window or ideally on an Ebike for quite a long time and having to go inland in places to cross a bridge.

- Method – ZRAD with attached GPS receiver running Geographic Location CC V2

  - ED beeps each time a GeoLoc Report is sent

  - Fully secure = DUT joined with S2 Auth so ZERO bits dropped to blink LED

- Goal:

  - Compare ZWLR and Z-Wave Mesh at very long ranges LoS

- Devices to test:

  - Controllers:

    - Mounted on a PVC pipe with rubber bands about 5’ above ground

    - ZRAD – different antennas

  - End Devices

    - Mounted on a PVC pipe with rubber bands and held at ~5’ above ground

    - ZRAD Battery powered with CR123A – SwitchOnOff which will blink LED

    - Homeseer PS100 PCB antenna battery powered but with special firmware

    - DK2603 Thunderboard with Antenna #1

    - XG28-EK2705A Explorer Kit with chip antenna

Future testing: Use Geographic Location CC to capture numerous data points with full 2D heat maps. A boat would also enable much wider range. Ideally offshore at Isle of Shoals but Great Bay or a large lake would work OK.

- Geographic Location CC loaded into a SwOnOff ZRAD

  - Read out the exact GPS location while the DUT is moving

  - Controller will GET the GPS location and store it with Tx power and stats

    - S2 Security encrypted = zero bits corrupted

    - SDK Retries are OK – no application retries

    - Quantitative threshold for communication vs. subjective (led blinking)

    - Store .csv file which can then be plotted on a map in excel

  - GPS sensor is the SparkFun XA1110 with a small antenna

    - Ensure the Blue LED is blinking indicating it has satellite lock

    - Should not be a problem since testing is outdoors in open space

- All trials are joined with S2 Authenticated requiring 100% perfect data or frame cannot be decrypted

- Planned Trials:

  - ZWLR testing:

  - ZRAD Controller ZWLR antenna #2 + ZRAD SwOnOff antenna #2

  - ZRAD Controller antenna #2 + ZRAD SwOnOff antenna #2 – classic ZW instead of ZWLR

  - Same as above but with poor antenna on ZRAD SwOnOff

  - TBZ Controller ZWLR + ZRAD SwOnOff antenna #2

  - ZG28 devkit ZWLR (chip antenna) + ZRAD SwOnOff antenna #2

  - UZB + ZRAD SwOnOff antenna #2

  - EU Testing:

  - ZRAD Controller EU +14dBm + ZRAD SwOnOff antenna #2 EU +14dBm

  - ZRAD Controller EU antenna #1 + off-the-shelf EU device

  - ZRAD Controller ZWLR antenna #1 + off-the-shelf ZWLR device (homeseer PS100) PCB antenna

  - ZRAD Controller ZWLR antenna #1 + other ZWLR device